NETC311: INTRODUCTION TO NETWORKS v7.0 (ITN)

Module 2: Basic Switch and End Device Configuration

CCNA 1: Introduction to Networks v7.0 — Identification Questionnaire

| Name: _________________________________________ | Date: ________________________ |
| --- | --- |
| Course & Year: NETC311 / Year III | Score: _________ / 104 |

GENERAL INSTRUCTIONS: This questionnaire contains 104 identification-type items based thoroughly on Module 2: Basic Switch and End Device Configuration. Read each statement carefully. Identify the correct term that completes the blank (____) by selecting the best answer among choices a, b, c, or d. Write the letter of your choice on the blank provided before each number or encircle the letter. A complete Answer Key and Study Reference is provided at the end of this document.

PART I: IDENTIFICATION QUESTIONNAIRE

_____ 1. The user interface of an operating system that allows users to request specific tasks through either CLI or GUI interfaces is called the ____.

a.) kernel                             b.) shell

c.) hardware                           d.) firmware

_____ 2. The core component of an operating system that communicates directly between computer hardware and software to manage hardware resources is the ____.

a.) shell                              b.) registry

c.) BIOS                               d.) kernel

_____ 3. The physical parts and underlying electronic circuitry of a computer or network device are collectively known as the ____.

a.) hardware                           b.) middleware

c.) software                           d.) firmware

_____ 4. A user-friendly operating system interface that allows users to interact with a system using graphical icons, menus, and windows is a ____.

a.) API

b.) Kernel shell

c.) CLI (Command Line Interface)

d.) GUI (Graphical User Interface)

_____ 5. Network operating systems on infrastructure devices like switches and routers are typically accessed and configured through a text-based ____.

a.) web wizard only

b.) touchscreen

c.) CLI (Command Line Interface)

d.) GUI

_____ 6. One reason network devices rely primarily on a CLI rather than a GUI is because GUIs consume extra resources and can fail, freeze, or ____.

a.) encrypt                            b.) re-route

c.) authenticate                       d.) crash

_____ 7. While a PC operating system enables users to use a mouse and view visual output, a CLI-based network operating system relies primarily on a technician using a ____.

a.) keyboard                           b.) graphics tablet

c.) stylus pen                         d.) touchpad

_____ 8. A physical management port on a Cisco network device used for out-of-band maintenance and initial device configuration is the ____.

a.) console port                       b.) AUX port

c.) VTY line                           d.) Ethernet port

_____ 9. The recommended method for remotely and securely connecting to the CLI of a Cisco device over a network is ____.

a.) Telnet                             b.) TFTP

c.) Secure Shell (SSH)                 d.) HTTP

_____ 10. An older remote terminal protocol that establishes a remote CLI connection over the network but transmits passwords and data in plaintext is ____.

a.) HTTPS                              b.) SFTP

c.) SSH                                d.) Telnet

_____ 11. Connecting to a network device via the console port is considered an out-of-band management method because it does not require active ____.

a.) network services                   b.) terminal software

c.) a serial cable                     d.) electrical power

_____ 12. Software applications such as PuTTY, Tera Term, and SecureCRT used by technicians to connect to network devices are known as ____.

a.) terminal emulation programs        b.) packet analyzers

c.) debuggers                          d.) compilers

_____ 13. The basic Cisco IOS command mode that allows access to only a limited number of basic monitoring commands is called ____.

a.) User EXEC mode                     b.) Line configuration mode

c.) Global configuration mode          d.) Privileged EXEC mode

_____ 14. On a Cisco IOS device named Switch, the User EXEC mode is identified by the prompt symbol ending with ____.

a.) >                                  b.) %

c.) $                                  d.) #

_____ 15. The Cisco IOS command mode that allows access to all device commands, monitoring features, and configuration modes is ____.

a.) Interface mode                     b.) Setup mode

c.) User EXEC mode                     d.) Privileged EXEC mode

_____ 16. On a Cisco IOS device named Switch, the Privileged EXEC mode is identified by the prompt ending with ____.

a.) #                                  b.) :

c.) ~                                  d.) >

_____ 17. The primary configuration mode used to configure device-wide settings and access specific subconfiguration modes is called ____.

a.) user EXEC mode

b.) line configuration mode

c.) interface configuration mode

d.) global configuration mode

_____ 18. The CLI prompt displayed when entering global configuration mode on a switch named Switch is ____.

a.) Switch(config-line)#               b.) Switch#

c.) Switch(config-if)#                 d.) Switch(config)#

_____ 19. The subconfiguration mode used to configure console, AUX, SSH, or Telnet access lines is called ____.

a.) routing configuration mode

b.) line configuration mode

c.) interface configuration mode

d.) vlan configuration mode

_____ 20. The CLI prompt displayed when configuring a console or VTY line on a switch is ____.

a.) Switch>                            b.) Switch(config-if)#

c.) Switch(config)#                    d.) Switch(config-line)#

_____ 21. The subconfiguration mode used to configure a switch port or router physical interface is called ____.

a.) system configuration mode

b.) line configuration mode

c.) interface configuration mode

d.) device configuration mode

_____ 22. The CLI prompt displayed when configuring a switch interface such as FastEthernet or VLAN 1 is ____.

a.) Switch(config-if)#                 b.) Switch(config)#

c.) Switch(config-line)#               d.) Switch#

_____ 23. To move from User EXEC mode to Privileged EXEC mode, the administrator enters the command ____.

a.) disable                            b.) enable

c.) exit                               d.) configure terminal

_____ 24. To move from Privileged EXEC mode down to User EXEC mode, enter the command ____.

a.) exit                               b.) disable

c.) logout                             d.) enable

_____ 25. To navigate from Privileged EXEC mode into global configuration mode, execute the command ____.

a.) configure terminal                 b.) enable config

c.) interface config                   d.) setup terminal

_____ 26. To return from global configuration mode back to Privileged EXEC mode, or from any subconfiguration mode back to global configuration mode, use the command ____.

a.) quit                               b.) back

c.) stop                               d.) exit

_____ 27. To exit from any subconfiguration mode directly back to Privileged EXEC mode in a single step, use the command ____.

a.) close                              b.) end

c.) return                             d.) exit

_____ 28. The keyboard shortcut that performs the exact same function as the 'end' command by returning directly to Privileged EXEC mode is ____.

a.) Ctrl+X                             b.) Ctrl+Z

c.) Ctrl+Shift+6                       d.) Ctrl+C

_____ 29. In a Cisco IOS command string, a predefined parameter defined by the operating system is known as a ____.

a.) constant                           b.) argument

c.) variable                           d.) keyword

_____ 30. In a Cisco IOS command string, a user-supplied parameter or variable (such as an IP address or password) is called an ____.

a.) delimiter                          b.) operand

c.) keyword                            d.) argument

_____ 31. In Cisco IOS command syntax documentation, text formatted in boldface indicates commands and keywords that must be entered ____.

a.) in uppercase only                  b.) as optional values

c.) literally as shown                 d.) as user-defined variables

_____ 32. In Cisco IOS command syntax documentation, text formatted in italics indicates an element for which the user must supply a ____.

a.) command prompt                     b.) bracket

c.) literal string                     d.) value

_____ 33. In Cisco command syntax conventions, elements enclosed within square brackets [ ] represent an ____ element.

a.) obsolete                           b.) optional

c.) required                           d.) mandatory

_____ 34. In Cisco command syntax conventions, elements enclosed within braces { } represent a ____ element.

a.) required                           b.) deprecated

c.) optional                           d.) commented

_____ 35. In Cisco command syntax conventions, a vertical line | within braces or brackets represents a ____.

a.) wildcard character

b.) logical AND condition

c.) choice between mutually exclusive elements

d.) pipe to file output

_____ 36. In Cisco syntax conventions, the pattern [x {y | z}] indicates a required choice within an ____ element.

a.) mandatory                          b.) optional

c.) administrative                     d.) encrypted

_____ 37. The Cisco IOS command used to verify layer 3 IP reachability to a destination device by sending ICMP echo requests is ____.

a.) telnet                             b.) ping

c.) traceroute                         d.) arp

_____ 38. The Cisco IOS command used to trace the path and display intermediate router hops to a remote host is ____.

a.) ping                               b.) traceroute

c.) ipconfig                           d.) netstat

_____ 39. Entering a question mark (?) at the CLI prompt to see which commands or arguments are available utilizes the feature called ____.

a.) auto-complete buffer               b.) command syntax check

c.) terminal logging                   d.) context-sensitive help

_____ 40. When an entered command is incomplete, ambiguous, or invalid, the IOS feature that provides feedback indicating the error is ____.

a.) command syntax check               b.) terminal history

c.) packet checker                     d.) context-sensitive help

_____ 41. In the Cisco CLI, entering a unique starting sequence of characters and pressing the ____ key will automatically complete the command.

a.) Tab                                b.) Esc

c.) Spacebar                           d.) Enter

_____ 42. In the Cisco CLI, commands can be shortened to the minimum number of characters that identify a ____ selection.

a.) unique                             b.) alphabetic

c.) numerical                          d.) case-sensitive

_____ 43. The keystroke used in the CLI editing buffer to erase the character to the left of the cursor is ____.

a.) Tab                                b.) Spacebar

c.) Delete                             d.) Backspace

_____ 44. The keyboard shortcut that moves the cursor one character to the left, identical to the Left Arrow key, is ____.

a.) Ctrl+A                             b.) Ctrl+B

c.) Ctrl+F                             d.) Ctrl+P

_____ 45. The keyboard shortcut that moves the cursor one character to the right, identical to the Right Arrow key, is ____.

a.) Ctrl+F                             b.) Ctrl+N

c.) Ctrl+E                             d.) Ctrl+B

_____ 46. The keyboard shortcut that recalls the previous command in the history buffer, identical to the Up Arrow key, is ____.

a.) Ctrl+U                             b.) Ctrl+N

c.) Ctrl+R                             d.) Ctrl+P

_____ 47. When command output produces more text than can be displayed in the terminal window, the IOS pauses and displays the ____ prompt.

a.) --Wait--                           b.) --Pause--

c.) --Continue--                       d.) --More--

_____ 48. While viewing output at the '--More--' prompt, pressing the ____ displays the next single line.

a.) Enter key                          b.) Shift key

c.) Tab key                            d.) Spacebar

_____ 49. While viewing output at the '--More--' prompt, pressing the ____ displays the next full screen.

a.) Tab key                            b.) Enter key

c.) Spacebar                           d.) Backspace

_____ 50. Pressing any alphanumeric key other than Enter or Spacebar while at the '--More--' prompt ends the display string and returns to ____.

a.) User EXEC mode                     b.) ROMmon mode

c.) Global configuration mode          d.) Privileged EXEC mode

_____ 51. When in any configuration mode, entering the shortcut ____ ends configuration mode and returns to privileged EXEC mode.

a.) Ctrl-D                             b.) Ctrl-A

c.) Ctrl-Shift-6                       d.) Ctrl-C

_____ 52. The all-purpose break sequence used to abort DNS lookups, traceroutes, and pings on Cisco devices is ____.

a.) Ctrl-Alt-Delete                    b.) Ctrl-C

c.) Ctrl-Shift-6                       d.) Ctrl-Z

_____ 53. The very first configuration command that should be applied to any Cisco device to provide a unique network identity is assigning a ____.

a.) hostname                           b.) subnet mask

c.) banner                             d.) IP address

_____ 54. By default, an unconfigured Cisco IOS switch is assigned the factory default hostname of ____.

a.) Default                            b.) Cisco

c.) Router                             d.) Switch

_____ 55. According to Cisco naming guidelines, a device hostname must start with a letter, contain no spaces, end with a letter or digit, and be less than ____ characters in length.

a.) 128                                b.) 64

c.) 32                                 d.) 256

_____ 56. To remove a configured hostname and return a switch prompt to its factory default name, enter the global command ____.

a.) no hostname                        b.) clear hostname

c.) erase hostname                     d.) default hostname

_____ 57. Cisco password guidelines recommend that passwords used for administrative device security should be more than ____ characters in length.

a.) six                                b.) eight

c.) sixteen                            d.) four

_____ 58. Simple passwords such as 'cisco' or 'class' used in lab environments are considered weak and easily guessable, and should be avoided in ____ environments.

a.) production                         b.) classroom

c.) simulation                         d.) virtual

_____ 59. To configure user EXEC password security for the physical management console, the administrator enters the global command ____.

a.) line terminal 0                    b.) line aux 0

c.) line vty 0                         d.) line console 0

_____ 60. In line configuration mode, after entering the 'password <string>' command, user EXEC access must be enabled using the command ____.

a.) enable                             b.) login

c.) activate                           d.) auth

_____ 61. To secure Privileged EXEC mode access with an encrypted, hashed password, the administrator uses the global command ____.

a.) encrypt enable                     b.) enable secret

c.) enable password                    d.) service password

_____ 62. Virtual lines that allow remote administrative access using Telnet or SSH to a Cisco device are called ____ lines.

a.) Ethernet                           b.) Console

c.) VTY                                d.) AUX

_____ 63. Many Cisco switches support up to 16 virtual terminal lines, which are configured concurrently using the command ____.

a.) line vty 0 15                      b.) line remote 0 15

c.) line console 0 15                  d.) line vty 1 16

_____ 64. By default, configuration files such as startup-config and running-config display configured passwords in ____.

a.) plaintext                          b.) MD5 hashes

c.) hexadecimal codes                  d.) SHA-256 ciphertext

_____ 65. To encrypt all plaintext passwords stored in the configuration files, execute the global configuration command ____.

a.) encrypt-passwords all              b.) crypto password-encrypt

c.) service password-encryption        d.) secure passwords enable

_____ 66. To verify that all configured passwords on the device are now encrypted, the administrator executes the Privileged EXEC command ____.

a.) show security-keys                 b.) show passwords

c.) show encryption                    d.) show running-config

_____ 67. A legal notification displayed to warn unauthorized personnel against attempting to access a device is configured using the global command ____.

a.) banner motd                        b.) login prompt

c.) legal notice                       d.) syslog warning

_____ 68. In the command 'banner motd # Unauthorized Access Prohibited #', the '#' character placed at the beginning and end of the message is called the ____ character.

a.) delimiting                         b.) control

c.) wildcard                           d.) escape

_____ 69. The saved configuration file stored in NVRAM that is loaded and executed when a Cisco device reboots is called ____.

a.) startup-config                     b.) running-config

c.) active-config                      d.) boot-config

_____ 70. The active configuration file stored in Random Access Memory (RAM) that immediately reflects changes made on the CLI is the ____.

a.) flash-config                       b.) startup-config

c.) nvram-config                       d.) running-config

_____ 71. Because system RAM is ____ memory, modifying the running configuration affects operation immediately, but changes are lost if power is lost.

a.) permanent                          b.) static

c.) non-volatile                       d.) volatile

_____ 72. The persistent storage location where the startup configuration file is preserved across reboots and power outages is ____.

a.) NVRAM                              b.) TFTP server

c.) ROM cache                          d.) RAM

_____ 73. To save all active modifications from volatile RAM to non-volatile NVRAM, enter the Privileged EXEC command ____.

a.) copy running-config startup-config

b.) save running-config startup-config

c.) commit startup-config

d.) write ram nvram

_____ 74. If undesirable changes have been made to the running configuration and have not yet been saved, the administrator can restore the previous saved configuration using the command ____.

a.) reload                             b.) reboot system

c.) reset factory                      d.) rollback

_____ 75. If unwanted configuration changes have already been saved to the startup configuration file, the administrator can clear NVRAM using the command ____.

a.) delete running-config              b.) reset startup-config

c.) clear nvram-all                    d.) erase startup-config

_____ 76. After clearing the startup configuration file from NVRAM with 'erase startup-config', the administrator must enter the ____ command to clear the running-config from RAM.

a.) shutdown                           b.) exit

c.) reload                             d.) erase running-config

_____ 77. When archiving a Cisco device configuration to a text file using terminal software such as PuTTY or Tera Term, the first step is to enable session ____.

a.) scripting                          b.) mirroring

c.) sniffing                           d.) logging

_____ 78. While session logging is active in terminal software, the administrator captures the active configuration by issuing the Privileged EXEC command 'show running-config' or ____.

a.) show flash                         b.) show interfaces

c.) show startup-config                d.) show memory

_____ 79. After capturing configuration output in terminal software, logging is turned off by choosing the session logging option ____.

a.) Disable                            b.) Stop

c.) None                               d.) Close

_____ 80. The primary means of enabling devices to locate one another and establish end-to-end communication on the Internet is using ____.

a.) MAC addresses only                 b.) port numbers only

c.) hostnames only                     d.) IP addresses

_____ 81. The structure of an IPv4 address is formatted as four decimal numbers between 0 and 255 separated by dots, known as ____.

a.) hexadecimal notation               b.) dotted decimal notation

c.) colon-hex format                   d.) binary octet notation

_____ 82. In an IPv4 address, each of the four decimal numbers must have a value between 0 and ____.

a.) 512                                b.) 255

c.) 127                                d.) 256

_____ 83. An IPv4 subnet mask is a 32-bit value that differentiates the network portion of the address from the ____ portion.

a.) interface                          b.) host

c.) broadcast                          d.) domain

_____ 84. The IP address of the local router interface that an end host uses to reach remote networks and the internet is the ____.

a.) subnet mask                        b.) default gateway

c.) broadcast address                  d.) DNS server

_____ 85. An IPv6 address has a total length of ____ bits.

a.) 256                                b.) 128

c.) 32                                 d.) 64

_____ 86. An IPv6 address is written as a sequence of hexadecimal values separated into groups of four digits by the symbol ____.

a.) colon (:)                          b.) hyphen (-)

c.) slash (/)                          d.) period (.)

_____ 87. In an IPv6 address, each individual hexadecimal digit represents ____ binary bits.

a.) 8                                  b.) 16

c.) 4                                  d.) 32

_____ 88. A complete IPv6 address contains a total of ____ hexadecimal digits.

a.) 16                                 b.) 32

c.) 128                                d.) 64

_____ 89. Unlike some command parameters, IPv6 addresses are not case-sensitive and can be written in either lowercase or ____.

a.) octal                              b.) uppercase

c.) binary                             d.) italics

_____ 90. Differences between network media types include transmission speed, cost, environment, and the distance the media can carry a ____.

a.) signal                             b.) VLAN

c.) gateway                            d.) protocol

_____ 91. End devices on a network require an IP address in order to communicate with other devices, which can be configured manually or automatically using ____.

a.) ARP                                b.) DHCP

c.) ICMP                               d.) DNS

_____ 92. To manually configure an IPv4 address on a Windows PC, a user opens Control Panel > Network Sharing Center > Change adapter settings, selects the adapter, and opens ____ Properties.

a.) Local Area Connection              b.) Administrative Tools

c.) Device Manager                     d.) Windows Defender

_____ 93. In Windows PC networking, manual IPv4 settings are entered within the Properties dialog of Internet Protocol Version 4 (____).

a.) UDP/IPv4                           b.) TCP/IPv4

c.) NETBIOS/IPv4                       d.) ICMP/IPv4

_____ 94. The network protocol that automatically assigns an IPv4 address, subnet mask, default gateway, and DNS servers to end devices is ____.

a.) DNS                                b.) NTP

c.) DHCP                               d.) SMTP

_____ 95. By default, end devices such as PCs are configured to obtain an IP address automatically using ____.

a.) DHCP                               b.) Static IP

c.) APIPA only                         d.) Manual assignment

_____ 96. In addition to DHCPv6, an IPv6 end device can automatically acquire dynamic address allocation using ____.

a.) NAT

b.) Proxy DNS

c.) SLAAC (Stateless Address Autoconfiguration)

d.) ARP

_____ 97. To enable remote management access (SSH or Telnet) to a Layer 2 switch across a network, an IP address must be configured on an ____.

a.) auxiliary line

b.) physical loopback cable

c.) console port

d.) SVI (Switch Virtual Interface)

_____ 98. The default Switch Virtual Interface present on Cisco switches used for remote IP management is ____.

a.) interface loopback 0               b.) interface fastethernet 0/1

c.) interface vlan 10                  d.) interface vlan 1

_____ 99. To configure an IP address on an SVI, the administrator enters the command 'ip address' followed by the ip-address and the ____.

a.) mac-address                        b.) default-gateway

c.) subnet-mask                        d.) dns-server

_____ 100. After configuring the IP address and subnet mask on an SVI, the interface must be administratively enabled using the command ____.

a.) no shutdown                        b.) start

c.) up                                 d.) enable interface

_____ 101. On a Windows PC command prompt, the command used to verify the current IP address, subnet mask, and default gateway configuration is ____.

a.) ifconfig                           b.) show ip

c.) netstat                            d.) ipconfig

_____ 102. On a Cisco switch CLI, the Privileged EXEC command used to display a concise summary of the status and IP configuration of all interfaces is ____.

a.) show ip route

b.) show interfaces status

c.) show running-config interface

d.) show ip interface brief

_____ 103. To navigate from global configuration mode into interface configuration mode for the management VLAN, enter the command ____.

a.) interface vlan 0                   b.) switchport vlan 1

c.) interface vlan 1                   d.) line vlan 1

_____ 104. The command mode accessed immediately when a user establishes a console connection or boots a Cisco switch without entering passwords is ____.

a.) Line configuration mode            b.) Privileged EXEC mode

c.) Global configuration mode          d.) User EXEC mode

PART II: ANSWER KEY & STUDY REFERENCE — MODULE 2: BASIC SWITCH AND END DEVICE CONFIGURATION

Official Solutions, Correct Terms, and Curriculum Concept References

| Item # | Key | Correct Identification Term | Curriculum Topic & Reference Explanation |
| --- | --- | --- | --- |
| 1 | b.) | shell | [2.1 Operating Systems] The shell is the user interface that allows users to request specific tasks from the computer via CLI or GUI. |
| 2 | d.) | kernel | [2.1 Operating Systems] The kernel communicates between the hardware and software of a computer and manages how resources are used. |
| 3 | a.) | hardware | [2.1 Operating Systems] Hardware represents the physical parts of a computer including underlying electronics. |
| 4 | d.) | GUI (Graphical User Interface) | [2.1 GUI] A GUI allows the user to interact with the system using graphical icons, menus, and windows. |
| 5 | c.) | CLI (Command Line Interface) | [2.1 GUI] Network devices are typically accessed through a CLI because GUIs can crash, fail, or consume unnecessary resources. |
| 6 | d.) | crash | [2.1 GUI] GUIs can fail, crash, or simply not operate as specified; network devices are typically accessed through CLI. |
| 7 | a.) | keyboard | [2.1 Purpose of an OS] A CLI-based network OS enables a network technician to use a keyboard to run CLI-based network programs and enter text commands. |
| 8 | a.) | console port | [2.1 Access Methods] The console port is a physical management port used to access a device for maintenance, such as initial configuration. |
| 9 | c.) | Secure Shell (SSH) | [2.1 Access Methods] SSH establishes a secure remote CLI connection through a virtual interface over a network with encryption. |
| 10 | d.) | Telnet | [2.1 Access Methods] Telnet establishes an insecure remote CLI connection where user authentication, passwords, and commands are sent in plaintext. |
| 11 | a.) | network services | [2.1 Access Methods] Console access does not require any network connectivity or IP configuration on the device. |
| 12 | a.) | terminal emulation programs | [2.1 Terminal Emulation Programs] Terminal emulation programs are used to connect to a network device by either a console port or SSH/Telnet. |
| 13 | a.) | User EXEC mode | [2.2 Primary Command Modes] User EXEC mode allows access to only a limited number of basic monitoring commands. |
| 14 | a.) | > | [2.2 Primary Command Modes] User EXEC mode is identified by the CLI prompt that ends with the > symbol (e.g., Switch>). |
| 15 | d.) | Privileged EXEC mode | [2.2 Primary Command Modes] Privileged EXEC mode allows access to all commands and features and is identified by the # prompt. |
| 16 | a.) | # | [2.2 Primary Command Modes] Privileged EXEC mode is identified by the CLI prompt ending with the # symbol (e.g., Switch#). |
| 17 | d.) | global configuration mode | [2.2 Configuration Modes] Global configuration mode is used to access configuration options on the device. |
| 18 | d.) | Switch(config)# | [2.2 Configuration Modes] Global configuration mode is designated by the prompt Switch(config)#. |
| 19 | b.) | line configuration mode | [2.2 Subconfiguration Modes] Line configuration mode is used to configure console, SSH, Telnet or AUX access. |
| 20 | d.) | Switch(config-line)# | [2.2 Subconfiguration Modes] Line configuration mode displays the prompt Switch(config-line)#. |
| 21 | c.) | interface configuration mode | [2.2 Subconfiguration Modes] Interface configuration mode is used to configure a switch port or router interface. |
| 22 | a.) | Switch(config-if)# | [2.2 Subconfiguration Modes] Interface configuration mode displays the prompt Switch(config-if)#. |
| 23 | b.) | enable | [2.2 Navigation Between Modes] The enable command transitions the CLI from User EXEC mode to Privileged EXEC mode. |
| 24 | b.) | disable | [2.2 Navigation Between Modes] The disable command returns the CLI from Privileged EXEC mode to User EXEC mode. |
| 25 | a.) | configure terminal | [2.2 Navigation Between Modes] The configure terminal command moves the device from Privileged EXEC into global configuration mode. |
| 26 | d.) | exit | [2.2 Navigation Between Modes] The exit command returns to the immediately preceding command mode. |
| 27 | b.) | end | [2.2 Navigation Between Modes] The end command returns directly to Privileged EXEC mode from any configuration mode. |
| 28 | b.) | Ctrl+Z | [2.2 Navigation Between Modes] The key combination Ctrl+Z ends any configuration mode and returns to privileged EXEC mode. |
| 29 | d.) | keyword | [2.3 Basic IOS Command Structure] A keyword is a specific parameter defined in the operating system (e.g., 'ip protocols'). |
| 30 | d.) | argument | [2.3 Basic IOS Command Structure] An argument is not predefined; it is a value or variable defined by the user (e.g., '192.168.10.5'). |
| 31 | c.) | literally as shown | [2.3 IOS Command Syntax Check] Boldface text indicates commands and keywords that you enter literally as shown. |
| 32 | d.) | value | [2.3 IOS Command Syntax Check] Italic text indicates arguments for which you supply values. |
| 33 | b.) | optional | [2.3 IOS Command Syntax Check] Square brackets [x] indicate an optional element (keyword or argument). |
| 34 | a.) | required | [2.3 IOS Command Syntax Check] Braces {x} indicate a required element (keyword or argument). |
| 35 | c.) | choice between mutually exclusive elements | [2.3 IOS Command Syntax Check] A vertical line ¦ indicates a required or optional choice between mutually exclusive elements. |
| 36 | b.) | optional | [2.3 IOS Command Syntax Check] Braces and vertical lines within square brackets indicate a required choice within an optional element. |
| 37 | b.) | ping | [2.3 IOS Command Syntax Check] The ping command sends ICMP echo requests to test connectivity to an IP address. |
| 38 | b.) | traceroute | [2.3 IOS Command Syntax Check] The traceroute command traces the route and lists each intermediate router hop to a destination. |
| 39 | d.) | context-sensitive help | [2.3 IOS Help Features] Context-sensitive help allows users to see available commands, keywords, and arguments by typing '?'. |
| 40 | a.) | command syntax check | [2.3 IOS Help Features] Command syntax check verifies that a valid command was entered and gives feedback if invalid. |
| 41 | a.) | Tab | [2.3 Hot Keys and Shortcuts] The Tab key completes a partial command name entry. |
| 42 | a.) | unique | [2.3 Hot Keys and Shortcuts] Commands and keywords can be shortened to the minimum number of characters that identify a unique selection. |
| 43 | d.) | Backspace | [2.3 Hot Keys and Shortcuts] Backspace erases the character to the left of the cursor. |
| 44 | b.) | Ctrl+B | [2.3 Hot Keys and Shortcuts] Left Arrow or Ctrl+B moves the cursor one character to the left. |
| 45 | a.) | Ctrl+F | [2.3 Hot Keys and Shortcuts] Right Arrow or Ctrl+F moves the cursor one character to the right. |
| 46 | d.) | Ctrl+P | [2.3 Hot Keys and Shortcuts] Up Arrow or Ctrl+P recalls commands in the history buffer, beginning with the most recent. |
| 47 | d.) | --More-- | [2.3 Hot Keys and Shortcuts] When output exceeds the terminal window, the IOS displays a '--More--' prompt. |
| 48 | a.) | Enter key | [2.3 Hot Keys and Shortcuts] At the '--More--' prompt, the Enter key displays the next line. |
| 49 | c.) | Spacebar | [2.3 Hot Keys and Shortcuts] At the '--More--' prompt, the Spacebar displays the next full screen. |
| 50 | d.) | Privileged EXEC mode | [2.3 Hot Keys and Shortcuts] Pressing any other key ends the display string, returning to privileged EXEC mode. |
| 51 | d.) | Ctrl-C | [2.3 Hot Keys and Shortcuts] Ctrl-C ends configuration mode and returns to privileged EXEC mode. |
| 52 | c.) | Ctrl-Shift-6 | [2.3 Hot Keys and Shortcuts] Ctrl-Shift-6 is the all-purpose break sequence used to abort DNS lookups, traceroutes, pings, etc. |
| 53 | a.) | hostname | [2.4 Device Names] The first configuration command on any device should be to give it a unique hostname. |
| 54 | d.) | Switch | [2.4 Device Names] By default, all Cisco IOS switches are assigned the factory default name 'Switch'. |
| 55 | b.) | 64 | [2.4 Device Names] Hostnames must start with a letter, have no spaces, end with letter/digit, and be less than 64 characters. |
| 56 | a.) | no hostname | [2.4 Device Names] To return the switch to the default prompt, use the 'no hostname' global config command. |
| 57 | b.) | eight | [2.4 Password Guidelines] Cisco recommends passwords that are more than eight characters in length. |
| 58 | a.) | production | [2.4 Password Guidelines] Simple passwords like 'cisco' or 'class' are weak and should be avoided in production environments. |
| 59 | d.) | line console 0 | [2.4 Configure Passwords] To secure user EXEC mode access via console, first enter line console 0 configuration mode. |
| 60 | b.) | login | [2.4 Configure Passwords] The 'login' command enables password checking upon connecting to the line. |
| 61 | b.) | enable secret | [2.4 Configure Passwords] The 'enable secret password' command provides encrypted password protection for privileged EXEC mode. |
| 62 | c.) | VTY | [2.4 Configure Passwords] VTY (Virtual Terminal) lines enable remote access using Telnet or SSH to the device. |
| 63 | a.) | line vty 0 15 | [2.4 Configure Passwords] Switches support 16 VTY lines numbered 0 to 15, entered using 'line vty 0 15'. |
| 64 | a.) | plaintext | [2.4 Encrypt Passwords] The startup-config and running-config files display most passwords in plaintext by default. |
| 65 | c.) | service password-encryption | [2.4 Encrypt Passwords] The 'service password-encryption' command prevents passwords from appearing as plaintext in config files. |
| 66 | d.) | show running-config | [2.4 Encrypt Passwords] Use 'show running-config' to verify that passwords in the active configuration are encrypted. |
| 67 | a.) | banner motd | [2.4 Banner Messages] A banner message of the day is configured using 'banner motd # message #' to warn unauthorized users. |
| 68 | a.) | delimiting | [2.4 Banner Messages] The character entered before and after the banner message is called the delimiting character. |
| 69 | a.) | startup-config | [2.5 Configuration Files] The startup-config is stored in NVRAM and contains commands executed upon reboot. |
| 70 | d.) | running-config | [2.5 Configuration Files] The running-config is stored in RAM and reflects the current, active device configuration. |
| 71 | d.) | volatile | [2.5 Configuration Files] RAM is volatile memory; it loses all of its contents when the device is powered off or restarted. |
| 72 | a.) | NVRAM | [2.5 Configuration Files] NVRAM (Non-Volatile RAM) does not lose its contents when the device is powered off. |
| 73 | a.) | copy running-config startup-config | [2.5 Configuration Files] The 'copy running-config startup-config' command saves the running configuration to NVRAM. |
| 74 | a.) | reload | [2.5 Alter the Running Configurations] Issuing 'reload' in Privileged EXEC mode reboots the device and reloads startup-config from NVRAM. |
| 75 | d.) | erase startup-config | [2.5 Alter the Running Configurations] The 'erase startup-config' command clears the saved configuration file from NVRAM. |
| 76 | c.) | reload | [2.5 Alter the Running Configurations] After erasing startup-config, reloading the device clears the running-config from RAM. |
| 77 | d.) | logging | [2.5 Capture Configuration to a Text File] Step 2 in capturing config is to enable session logging in the terminal emulation software. |
| 78 | c.) | show startup-config | [2.5 Capture Configuration to a Text File] Executing 'show running-config' or 'show startup-config' prints the configuration into the log file. |
| 79 | c.) | None | [2.5 Capture Configuration to a Text File] Session logging is disabled in PuTTY by choosing the 'None' session logging option. |
| 80 | d.) | IP addresses | [2.6 IP Addresses] IP addresses are the primary means enabling devices to locate one another and establish communication. |
| 81 | b.) | dotted decimal notation | [2.6 IP Addresses] The structure of an IPv4 address is called dotted decimal notation. |
| 82 | b.) | 255 | [2.6 IP Addresses] Each decimal octet in an IPv4 address ranges from 0 to 255. |
| 83 | b.) | host | [2.6 IP Addresses] The subnet mask differentiates the network portion of the address from the host portion. |
| 84 | b.) | default gateway | [2.6 IP Addresses] The default gateway is the IP address of the router used by the host to access remote networks. |
| 85 | b.) | 128 | [2.6 IP Addresses] IPv6 addresses are 128 bits in length. |
| 86 | a.) | colon (:) | [2.6 IP Addresses] Groups of four hexadecimal digits in IPv6 are separated by a colon ':'. |
| 87 | c.) | 4 | [2.6 IP Addresses] Every four bits is represented by a single hexadecimal digit. |
| 88 | b.) | 32 | [2.6 IP Addresses] A 128-bit IPv6 address consists of 32 hexadecimal values (128 / 4 = 32). |
| 89 | b.) | uppercase | [2.6 IP Addresses] IPv6 addresses are not case-sensitive and can be written in lowercase or uppercase. |
| 90 | a.) | signal | [2.6 Interfaces and Ports] Differences include distance the media can successfully carry a signal, speed, environment, and cost. |
| 91 | b.) | DHCP | [2.7 Manual IP Address Configuration] IPv4 address information can be entered manually or automatically using DHCP. |
| 92 | a.) | Local Area Connection | [2.7 Manual IP Address Configuration] Right-clicking the adapter opens the Local Area Connection Properties window. |
| 93 | b.) | TCP/IPv4 | [2.7 Manual IP Address Configuration] Settings are entered in the Internet Protocol Version 4 (TCP/IPv4) Properties window. |
| 94 | c.) | DHCP | [2.7 Automatic IP Address Configuration] DHCP (Dynamic Host Configuration Protocol) enables automatic IPv4 address configuration. |
| 95 | a.) | DHCP | [2.7 Automatic IP Address Configuration] End devices are typically by default using DHCP for automatic IPv4 address configuration. |
| 96 | c.) | SLAAC (Stateless Address Autoconfiguration) | [2.7 Automatic IP Address Configuration] IPv6 uses DHCPv6 and SLAAC (Stateless Address Autoconfiguration) for dynamic address allocation. |
| 97 | d.) | SVI (Switch Virtual Interface) | [2.7 Switch Virtual Interface Configuration] To access the switch remotely, an IP address and subnet mask must be configured on the SVI. |
| 98 | d.) | interface vlan 1 | [2.7 Switch Virtual Interface Configuration] On a Cisco switch, the default virtual management interface is interface vlan 1. |
| 99 | c.) | subnet-mask | [2.7 Switch Virtual Interface Configuration] The syntax is: 'ip address ip-address subnet-mask'. |
| 100 | a.) | no shutdown | [2.7 Switch Virtual Interface Configuration] The virtual interface is enabled using the 'no shutdown' interface command. |
| 101 | d.) | ipconfig | [2.9 Module Practice and Quiz / Commands] The 'ipconfig' command displays the IP configuration on a Windows PC. |
| 102 | d.) | show ip interface brief | [2.9 Module Practice and Quiz / Commands] 'show ip interface brief' (or 'show ip int brief') lists interfaces, IP addresses, and line status. |
| 103 | c.) | interface vlan 1 | [2.7 Switch Virtual Interface Configuration] The command 'interface vlan 1' enters SVI configuration mode. |
| 104 | d.) | User EXEC mode | [2.2 Primary Command Modes] A user lands in User EXEC mode (prompt '>') upon initial connection. |